Laboratory Exercise 11

Interrupts & IO programming

Assignment 1

# Source code đã chỉnh sửa

.data

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.text

main:	li	$t1, IN_ADRESS_HEXA_KEYBOARD

li	$t2, OUT_ADRESS_HEXA_KEYBOARD

li 	$t3, 0x01		# check row 4 with key C, D, E, F

li	$t4, 0x08

polling:

sb	$t3, 0($t1)

lb	$a0, 0($t2)

print:

li	$v0, 34

syscall

sleep:

li	$a0, 100

li	$v0, 32

syscall

back_to_polling:

beq	$t3, $t4, reset

sll	$t3, $t3, 1

j	polling

reset:

li	$t3, 0x01

jal	polling

# Demo

- Row 1

- Row 2

- Row 3

- Row 4

Assignment 2

# Source Code

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.data

Message: .asciiz "Oh my god. Someone's presed a button.\n"

.text

main:

li 	$t1, IN_ADRESS_HEXA_KEYBOARD

li 	$t3, 0x80 		# bit 7 of = 1 to enable interrupt

sb    	$t3, 0($t1)

Loop:

nop

nop

nop

nop

b      Loop

end_main:

.ktext 0x80000180

IntSR:

addi 	$v0, $zero, 4   # show message

la 	$a0, Message

syscall

next_pc:

mfc0	$at, $14

addi	$at, $at, 4

mtc0	$at, $14

return:

eret

# Demo

# Giải thích:

- Ấn một kí tự bất kỳ bằng thiết bị Digital Lab Sim thì message sẽ được in ra màn hình.

Assignment 3

# Source code đã sửa đổi

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.data

Message: .asciiz "Key scan code "

.text

main:

li 	$t1, IN_ADRESS_HEXA_KEYBOARD

li 	$t3, 0x80 		# bit 7 = 1 to enable

li	$t4, 1

sb 	$t3, 0($t1)

xor 	$s0, $s0, $s0

Loop:	addi	$s0, $s0, 1

prn_seq:

addi	$v0, $zero, 1

add	$a0, $s0, $zero

syscall

prn_eol:

addi	$v0, $zero, 11

li	$a0, '\n'

syscall

sleep:

addi	$v0, $zero, 32

li	$a0, 300

syscall

nop

b	Loop

end_main:

.ktext 0x80000180

IntSR:

addi 	$sp, $sp, 4

sw 	$ra, 0($sp)

addi 	$sp, $sp, 4

sw 	$at, 0($sp)

addi 	$sp, $sp, 4

sw 	$v0, 0($sp)

addi 	$sp, $sp, 4

sw 	$a0, 0($sp)

addi 	$sp, $sp, 4

sw 	$t1, 0($sp)

addi 	$sp, $sp, 4

sw 	$t3, 0($sp)

prn_msg:

addi 	$v0, $zero, 4

la 	$a0, Message

syscall

get_cod:

li 	$t1, IN_ADRESS_HEXA_KEYBOARD

li 	$t3, 0x80

add	$t3, $t3, $t4

sb 	$t3, 0($t1)

li 	$t1, OUT_ADRESS_HEXA_KEYBOARD

lb 	$a0, 0($t1)

beq	$a0, $zero, continue		# Nếu output = 0 thì chuyển qua 						# hàng khác

j	prn_cod				# Output # 0  kết quả trên 						# hàng đó

continue:

sll	$t4, $t4, 1			# tăng index hàng thêm 1

beq	$t4, 8, prn_cod			# 80 + i*2 = hexa của hàng

j	get_cod				# tiếp theo

prn_cod:

li 	$v0, 34

syscall

li 	$v0, 11

li 	$a0, '\n'

syscall

next_pc:

mfc0 	$at, $14

addi 	$at, $at, 4

mtc0 	$at, $14

restore:

lw	$t3, 0($sp)

addi	$sp, $sp, -4

lw	$t1, 0($sp)

addi	$sp, $sp, -4

lw	$a0, 0($sp)

addi	$sp, $sp, -4

lw	$v0, 0($sp)

addi	$sp, $sp, -4

lw	$ra, 0($sp)

addi	$sp, $sp, -4

lw	$ra, 0($sp)

addi	$sp, $sp, -4

return: eret

Sửa code hàm get_cod

- Row 1

- Row 2

- Row 3

- Row 4

Assignment 4

# Source Code

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv COUNTER         0xFFFF0013

.eqv MASK_CAUSE_COUNTER 0x00000400

.eqv MASK_CAUSE_KEYMATRIX 0x00000800

.data

msg_keypress: .asciiz "Someone has pressed a key!\n"

msg_counter: .asciiz "Time inteval!\n"

.text

main:

li 	$t1, IN_ADRESS_HEXA_KEYBOARD

li 	$t3, 0x80 		# bit 7 = 1 to enable

sb 	$t3, 0($t1)

li 	$t1, COUNTER

sb 	$t1, 0($t1)

Loop:

nop

nop

nop

sleep:	addi 	$v0, $zero, 32

li	$a0, 200

syscall

nop

b 	Loop

end_main:

.ktext 0x80000180

IntSR:

dis_int:

li 	$t1, COUNTER 		# BUG: must disable with Time Counter

sb 	$zero, 0($t1)

get_caus:

mfc0 	$t1, $13

IsCount:

li	$t2, MASK_CAUSE_COUNTER

and 	$at, $t1, $t2

beq 	$at, $t2, Counter_Intr

IsKeyMa:

li 	$t2, MASK_CAUSE_KEYMATRIX

and	$at, $t1, $t2

beq 	$at, $t2, Keymatrix_Intr

others: j 	end_process

Keymatrix_Intr:

li 	$v0, 4 			# Processing Key Matrix Interrupt

la 	$a0, msg_keypress

syscall

j 	end_process

Counter_Intr:

li 	$v0, 4 			# Processing Counter Interrupt

la 	$a0, msg_counter

syscall

j 	end_process

end_process:

mtc0 	$zero, $13

en_int:

li 	$t1, COUNTER

sb 	$t1, 0($t1)

next_pc:

mfc0 	$at, $14

addi	$at, $at, 4

mtc0	$at, $14

return:	eret

# Demo

# Giải thích:

- Vòng lặp mãi mãi chỉ kết thúc nếu người dùng ấn dừng.

- Sau mỗi khoảng thời gian ngắn thì message “Time interval!” sẽ được in ra console.

- Tuy nhiên nếu người dùng nhập số bằng thiết bị Digital Lab Sim thì message “Some one has pressed a key!” sẽ được in ra console.

Assignment 5

# Source Code

.eqv KEY_CODE   0xFFFF0004

.eqv KEY_READY  0xFFFF0000

.eqv DISPLAY_CODE 0xFFFF000C

.eqv DISPLAY_READY 0xFFFF0008

.eqv MASK_CAUSE_KEYBOARD 0x0000034

.text

li   	$k0, KEY_CODE

li   	$k1, KEY_READY

li   	$s0, DISPLAY_CODE

li	$s1, DISPLAY_READY

loop: 	nop

WaitForKey:

lw	$t1, 0($k1)

beq 	$t1, $zero, WaitForKey

MakeIntR:

teqi 	$t1, 1

j 	loop

.ktext 0x80000180

get_caus:

mfc0 	$t1, $13

IsCount:

li 	$t2, MASK_CAUSE_KEYBOARD

and   	$at, $t1, $t2

beq   	$at, $t2, Counter_Keyboard

j    	end_process

Counter_Keyboard:

ReadKey:

lw 	$t0, 0($k0)

WaitForDis:

lw	$t2, 0($s1)

beq	$t2, $zero, WaitForDis

Encrypt:

addi	$t0, $t0, 1

ShowKey:

sw 	$t0, 0($s0)

nop

end_process:

next_pc:

mfc0 	$at, $14

addi    $at, $at, 4

mtc0 	$at, $14

return: eret

# Demo

# Giải thích

- Chương trình chỉ kết thúc khi người dùng ấn dừng.

- Các kí tự nhập vào sẽ được tính bằng mã ASCII của kí tự đó + 1.